Service item 1 means an ‘A’ service

Service item 2 means ‘B’ service.

Service Item 3 is due on every service as it is engine oil and filter change.

The additional service items so far that i’ve come up against:

Service 4 – Brake fluid change

Service 8 – Air filter replacement. Sometimes diff oil (check service sheet)

Service 10 – Sunroof guide rails clean / regrease

Service 11 – Fuel filter replacement

Service 12 – Replace spark plugs

Service 13 – Engine coolant renewal

Service 15 – Towing hitch lubrication

Service 20 – Gearbox oil change / filter renewal

Service 24 – Cambelt replacement / Waterpump replace

Here are the service scopes for the C-Class W204 and therefore for all other cars (>2004).

- Service 1: Minor basic service (Service A)

- Service 2: Major basic service (Service B)

- Service 3: Engine oil + filter

- Service 4: Brake fluid

- Service 5: Combination filter

- Service 6: Dust filter (included in Service B)

- Service 7: Activated charcoal filter

- Service 8: Air filter

- Service 9: Tilting/Sliding roof

- Service 10: Panoramic sliding roof

- Service 11: fuel filter

- Service 12: spark plugs

- Service 13: coolant

- Service 14: main coolant circuit

- Service 15: trailer hitch

- Service 16: parking brake

- Service 17: emergency call system

- Service 18: brake algorithm

- Service 19: diesel particulate filter

- Service 20: automatic transmission oil and filter

A – 505 – Service items 1, 3

A1 – D0D – Service items 1, 3, 4

A2

A3 – 550A – Service items 1, 3, 8, 11

A4 – D50J – Service items 1, 3, 8, 10, 11

A5

A6

A7 – G50M – Service items 1, 3, 20

A8 – Q50V – Service items 1, 3, 10, 20

A9 – GD0V – Service items 1, 3, 4, 20

A0

AC – QD051 – Service items 1, 3, 4, 10, 20

AH – 801050E – Service items 1, 3, 13, 24

AG – VD0A1 – Service items 1, 3, 4, 8, 10, 11, 20

AK – 10D0E – Service items 1, 3, 1

B – 606 – Service items 2, 3

B1 – E0E – Service items 2, 3, 4

B2 – 8E0N – Service items 2, 3, 4, 10

B3 – 560B – Service items 2, 3, 8, 11

B4 – B60H – Service items 2, 3, 8, 10, 11

B5 – 5E0K – Service items 2, 3, 4, 8, 11

B5 – 3E0H – Service items 2, 3, 4, 8, 12

B6 – DE0T – Service items 2, 3, 4, 8, 10, 11

B7 – G60N – Service items 2, 3, 20

B8

B9 – GE0W – Service items 2, 3, 4, 20

B0 – 1607 – Service items 2, 3, 8

BE – V6031 Service items 2, 3, 8, 10, 11, 20

BK – (AWAITING!) – Service items 2, 3, 4, 13 (i think!)

BS – 1Q60X – Service items 2, 3, 10, 13, 20