TO REPLACE REAR PADS on 2021 E450

1 – open hood and leave cap loose and then close hood

– get in car and close door, turn car on to full ignition 2 and release the EPBRAKE. Make sure the brake icon is off the dash. Shut car off then MAKE SURE YOUR FOOT IS OFF THE BRAKE PEDAL!!  press ignition to 1 position only, pull EPB handle again to be sure brake is off.  While in ignition 1 position, make sure odometer/mileage is on the middle of the screen.

Now here’s the sticky part,  hold down the Home button for 5 seconds, release, and then hold down the OK (4arrow) button for at least 20 seconds. Hopefully you will see the ‘Workshop Tools’ menu. If not, then try try again, maybe holding the OK button even longer

4 – go to ‘brake pad replacement’, press OK, then you see ‘Move to fitting Position’, press OK, you’ll hear the caliper noise momentarily with ‘Moving to Fitting Position’ on the screen,  just wait for it to finish! This process moves the calipers into the housing to allow pad removal and replacement.

5 – you now will see ‘Fitting Position Reached’ . DO NOT PRESS OK yet! Leave on the dash screen. Now take all the precautions you would when changing a tire. Loosen the wheel bolts just a bit til you raise the car and further remove the wheel bolts entirely. You can now remove the tire. Now go to remove the caliper housing. You will need a E12 Star socket which is used to remove the brake wear sensor housing as well as a ½” and 17mm wrenches to remove the caliper housing. There is a youtube video of a 2017 S Class 550 rear brake pad replacement. I only mention that so you can see where the wrenches are applied. You may want to have a small plastic bucket to rest the caliper on while working on the pads. The 2021 rear caliper also has a few cables to move out of the harness and a black rectangle plastic electrical plug to disconnect. GENTLY take the wear sensor out of the pad. Now remove the old pads.

6 – Place the new pads in being careful to make sure the clips are place in properly and the pads sit flat to the rotor and reconnect the wear sensor into the inner pad sensor hole.

ATTENTION!!! I bought the ‘Powerstop’ brand of pads and on the right side of the car, you may encounter having a tough time to put the caliper housing back on. This was due to the pad on the outside having the ‘two-prong’ sensor housing not lined up with the other inside pad ‘prongs’. I just hacksawed the prongs off. This was not a problem on the left hand side of the car, don’t ask me why that is.

7 – re-assemble caliper housing, the cables in the clip and the rectangle electrical plug.

8 – QUICKLY GO BACK TO THE CAR AND CLOSE DOOR AND TAP THE ‘OK’ BUTTON TO exit the Fitting Position. You will now hear the piston sound again and WAIT for the ‘Exiting Fitting Position’ to finish.

You now will see ‘Fitting Position has been exited’ and you hit OK

9 – you can now exit the Brake Pad replacement tool and go back to TRIP on the screen

10 – NOW OPEN HOOD and close the fluid container.

TO REPLACE ROTORS

You must do all of the above pad replac procedure

After rotor is removed, brush and clean hub, regrease the seat, place rotor on and re-screw with 10 Newton Meters