SERVICE SHEET for Mercedes-Benz Maintenance System (USA)

AP00.20-U-1208DN

# Model Year 2018

USA	Model 213

Customer:	Repair order number:

Tire Rotation

(non-staggered width wheel setup only)

A 5,000-mile recommended tire rotation interval will likely necessitate tire rotation at least once in between each 10,000-mile maintenance service and at every 10,000-mile maintenance service.

Tires rotated	YES	NO

Note: For any condition found while performing the checks or inspections included in this maintenance sheet, determine the cause and perform the repair using a separate repair order.

Current mileage reading:	Oil grade/viscosity:

Additional notes:

Service work to be performed as advised by service advisor:

Completion date:	Confirm work has been performed by signing

below:

P-2374-213-18A (11/17)

Dealer Workshop Services

Mercedes-Benz USA, LLC  Reproduction or translation in whole or in part is not permitted

without written authorization from the publisher.

www.MBUSA.com	1-800-FOR-MERCedes

www.startekinfo.com

1

| Required Service Work Performed (check if performed) | miles (thousands) years | 2 | 10 1 | 20 2 | 30 3 | 40 4 | 50 5 | 60 6 | 70 7 | 80 8 | 90 9 | 100 10 | 110 11 | 120 12 | 130 13 | 140 14 | 150 15 | Not OK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset maintenance service indicator in instrument cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP00.20-P-0043J |
| Replace activated charcoal combination filter behind glovebox |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP83.00-P-8384B |
| Replace dust filter behind plastic cowl in engine compartment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP83.00-P-8381C |
| Perform brake test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP42.00-P-4290BA |
| Engine – oil and filter change |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP18.00-P-0101DD |
| engine 274.920 except CODE M005 (4Matic) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP18.00-P-0101CWI |
| engine 274.920 with CODE M005 (4Matic) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP18.00-P-0101CWE |
| engine 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP18.00-P-0101CWZ |
| Check condition of poly-V-belt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP13.22-P-1352D |
| engine 274, 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP13.22-P-1352EW |
| Check thickness of brake pads front and rear |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP42.10-P-4252E |
| Check and correct all tire air pressures, including spare where applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP40.15-P-4061A |
| Check visible parts in engine compartment for damage and leaks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP00.20-P-0055E |
| Check underside of vehicle for damage and leaks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP00.20-P-0054I |
| Inspect front axle ball joints and rubber boots |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP33.10-P-3358A |
| Inspect driveshaft flexible discs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP41.10-P-4152EW |
| Inspect rear axle and rear axle ball joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP35.20-P-3552A |
| Check condition of steering and tie rods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP46.00-P-4651A |
| Check condition of suspension struts or shock absorbers and springs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP32.25-P-3255A |
| Check catch, safety catch, and hinges on engine hood for proper operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean visible area of water deflector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.35-P-8232A |
| Check coolant level and correct if necessary observe correct coolant composition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP20.30-P-2049U |
| Check windshield washer system fluid, correct if necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.35-P-8210C |
| Check brake fluid level and correct if necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP42.10-P-4210B |
| Replace brake fluid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP42.00-P-4280C |
| Check first aid kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP68.00-P-6850EW |
| Reset maintenance service indicator in instrument cluster reminder - if not yet performed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP00.20-P-0043J |
| Required Additional Work miles (thousands) Performed years (check if performed) |  | 2 | 10 1 | 20 2 | 30 3 | 40 4 | 50 5 | 60 6 | 70 7 | 80 8 | 90 9 | 100 10 | 110 11 | 120 12 | 130 13 | 140 14 | 150 15 | Not OK |
| Rear Axle differential fluid replacement with CODE 467 (electronic locking differential) with CODE 471 (mechanical locking differential) with CODE P30 (AMG performance package) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with CODE 471 (mechanical locking differential) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP35.00-P-3501CMG |
| with CODE 467 (electronic locking differential) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP35.00-P-3501CMH |

| Required Additional Work miles (thousands) Performed years (check if performed) | 2 | 10 1 | 20 2 | 30 3 | 40 4 | 50 5 | 60 6 | 70 7 | 80 8 | 90 9 | 100 10 | 110 11 | 120 12 | 130 13 | 140 14 | 150 15 | Not OK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Replace air filter engine 177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP09.10-P-0980K |
| Replace air filter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 274, 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 274 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP09.10-P-0980CWG |
| engine 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP09.10-P-0980E |
| Replace spark plugs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP15.10-P-1580C |
| engine 274 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP15.10-P-1580CWF |
| engine 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP15.10-P-1580D |
| Automatic transmission - oil and filter change engine 177 with transmission 725.09 (AMG models) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP27.00-P-2702B |
| Automatic transmission - oil and filter change engine 274, 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP27.00-P-2702B |
| Replace fuel filter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP47.20-P-0780D |
| Replace main circuit coolant observe correct coolant composition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP20.00-P-2082G |
| Replace low-temperature circuit coolant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| engine 177 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP20.00-P-2083C |
| engine 276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP20.00-P-2083I |
| Recommended Service miles (thousands) Work Performed years (check if performed) | 2 | 10 1 | 20 2 | 30 3 | 40 4 | 50 5 | 60 6 | 70 7 | 80 8 | 90 9 | 100 10 | 110 11 | 120 12 | 130 13 | 140 14 | 150 15 | Not OK |
| Check horn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check warning/indicator lamps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP54.30-P-5451HA |
| Check illumination and interior lighting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP54.30-P-5453E |
| Activate Tire Pressure Monitoring System Refer to vehicle Operator’s Manual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check windshield and headlamp washer system operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.35-P-8252D |
| Replace wiper blades |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check, clean and lubricate Panoramic sliding roof guide mechanism with CODE 413 (Automatic panoramic sliding sunroof) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP77.20-P-7732A |
| Check all exterior lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.10-P-8252EW |
| Check operation of headlamp range adjustment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.10-P-8250EW |
| Check headlamp setting, correct if necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP82.10-P-8260J |
| Check for body paint damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP60.00-P-0050I |
| Check trunk / cargo area light |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Replace battery for KEYLESS-GO key with CODE 889 (KEYLESS-GO) or with CODE 893 (KEYLESS-GO start function) After consultation with customer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check condition and operation of trailer hitch with CODE 550 (Trailer hitch) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP31.11-P-3152A |
| Check TIREFIT tire sealant expiration date (where applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP40.10-P-4054R |
| Check battery. Attach printout to sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP54.10-P-5449U |
| Tire rotation (where applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Recommended Service miles (thousands) Work Performed years (check if performed) | 2 | 10 1 | 20 2 | 30 3 | 40 4 | 50 5 | 60 6 | 70 7 | 80 8 | 90 9 | 100 10 | 110 11 | 120 12 | 130 13 | 140 14 | 150 15 | Not OK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check seatbelts and buckles for proper operation or damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check thickness of brake pads and discs, front and rear wheels removed pads discs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP42.10-P-4252E AP42.10-P-4256EW |
| Visually inspect brake system components, lines, hoses, calipers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect tires for damage and measure remaining tread life |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP40.10-P-4051EW |
| Check chassis and load bearing body components for damage and corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check ex-factory driving aids (where applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AP86.20-P-8649U |
| Check navigation system version for update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High Mileage Required Service Work at 160,000 miles or 16 years; then every 20,000 miles or 2 years Plus any applicable additional work listed at the end of the service sheet Performed Not OK (check if performed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset maintenance service indicator in instrument cluster AP00.20-P-0043J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Replace activated charcoal combination filter AP83.00-P-8384B behind glovebox |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Replace dust filter AP83.00-P-8381C behind plastic cowl in engine compartment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Perform brake test AP42.00-P-4290BA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine – oil and filter change engine 177 AP18.00-P-0101DD engine 274.920 except CODE M005 (4Matic) AP18.00-P-0101CWI engine 274.920 with CODE M005 (4Matic) AP18.00-P-0101CWE engine 276 AP18.00-P-0101CWZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check condition of poly-V-belt engine 177 AP13.22-P-1352D engine 274, 276 AP13.22-P-1352EW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check thickness of brake pads front and rear AP42.10-P-4252E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check and correct all tire air pressures, including spare where applicable AP40.15-P-4061A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check visible parts in engine compartment for damage and leaks AP00.20-P-0055E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check underside of vehicle for damage and leaks AP00.20-P-0054I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect front axle ball joints and rubber boots AP33.10-P-3358A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect driveshaft flexible discs AP41.10-P-4152EW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect rear axle and rear axle ball joints AP35.20-P-3552A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check condition of steering and tie rods AP46.00-P-4651A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check condition of suspension struts or shock absorbers and springs AP32.25-P-3255A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check catch, safety catch, and hinges on engine hood for proper operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean visible area of water deflector AP82.35-P-8232A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check coolant level and correct if necessary AP20.30-P-2049U observe correct coolant composition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check windshield washer system fluid, correct if necessary AP82.35-P-8210C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check brake fluid level and correct if necessary AP42.10-P-4210B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Replace brake fluid | AP42.00-P-4280C |
| --- | --- |
| Check first aid kit | AP68.00-P-6850EW |
| Reset maintenance service indicator in instrument cluster reminder - if not yet performed | AP00.20-P-0043J |
| High Mileage Recommended Service Work at 160,000 miles or 16 years; then every 20,000 miles or 2 years Plus any applicable additional work listed at the end of the service sheet Performed Not OK (check if performed) |  |
| Check horn |  |
| Check warning/indicator lamps | AP54.30-P-5451HA |
| Check illumination and interior lighting | AP54.30-P-5453E |
| Activate Tire Pressure Monitoring System Refer to vehicle Operator’s Manual |  |
| Check windshield and headlamp washer system operation | AP82.35-P-8252D |
| Replace wiper blades |  |
| Check, clean and lubricate Panoramic sliding roof guide mechanism with CODE 413 (Automatic panoramic sliding sunroof) | AP77.20-P-7732A |
| Check all exterior lights | AP82.10-P-8252EW |
| Check operation of headlamp range adjustment | AP82.10-P-8250EW |
| Check headlamp setting, correct if necessary | AP82.10-P-8260J |
| Check for body paint damage | AP60.00-P-0050I |
| Check trunk / cargo area light |  |
| Replace battery for KEYLESS-GO key with CODE 889 (KEYLESS-GO) or with CODE 893 (KEYLESS-GO start function) After consultation with customer |  |
| Check condition and operation of trailer hitch with CODE 550 (Trailer hitch) | AP31.11-P-3152A |
| Check TIREFIT tire sealant expiration date (where applicable) | AP40.10-P-4054R |
| Check battery. Attach printout to sheet | AP54.10-P-5449U |
| Tire rotation (where applicable) |  |
| Check seatbelts and buckles for proper operation or damage |  |
| Check thickness of brake pads and discs, front and rear wheels removed pads discs | AP42.10-P-4252E AP42.10-P-4256EW |
| Visually inspect brake system components, lines, hoses, calipers |  |
| Inspect tires for damage and measure remaining tread life | AP40.10-P-4051EW |
| Check chassis and load bearing body components for damage and corrosion |  |
| Check ex-factory driving aids (where applicable) | AP86.20-P-8649U |
| Check navigation system version for update |  |
| High Mileage Required Service Work at 170,000 miles or 17 years; then every 20,000 miles or 2 years Plus any applicable additional work listed at the end of the service sheet Performed Not OK (check if performed) |  |
| Reset maintenance service indicator in instrument cluster | AP00.20-P-0043J |
| Engine – oil and filter change engine 177 engine 274.920 except CODE M005 (4Matic) engine 274.920 with CODE M005 (4Matic) engine 276 | AP18.00-P-0101DD AP18.00-P-0101CWI AP18.00-P-0101CWE AP18.00-P-0101CWZ |

| Check thickness of brake pads front and rear |  | AP42.10-P-4252E |
| --- | --- | --- |
| Check and correct all tire air pressures, including spare where applicable |  | AP40.15-P-4061A |
| Check catch, safety catch, and hinges on engine hood for proper operation |  |  |
| Check coolant level and correct if necessary observe correct coolant composition |  | AP20.30-P-2049U |
| Check windshield washer system fluid, correct if necessary |  | AP82.35-P-8210C |
| Check brake fluid level and correct if necessary |  | AP42.10-P-4210B |
| Check first aid kit |  | AP68.00-P-6850EW |
| Reset maintenance service indicator in instrument cluster reminder - if not yet performed |  | AP00.20-P-0043J |
| High Mileage Recommended Service Work at 170,000 miles or 17 years; then every 20,000 miles or 2 years Plus any applicable additional work listed at the end of the service sheet Performed Not OK (check if performed) |  |  |
| Check horn |  |  |
| Check warning/indicator lamps |  | AP54.30-P-5451HA |
| Check illumination and interior lighting |  | AP54.30-P-5453E |
| Activate Tire Pressure Monitoring System Refer to vehicle Operator’s Manual |  |  |
| Check windshield and headlamp washer system operation |  | AP82.35-P-8252D |
| Replace wiper blades |  |  |
| Check all exterior lights |  | AP82.10-P-8252EW |
| Check trunk / cargo area light |  |  |
| Replace battery for KEYLESS-GO key with CODE 889 (KEYLESS-GO) or with CODE 893 (KEYLESS-GO start function) After consultation with customer |  |  |
| Check TIREFIT tire sealant expiration date (where applicable) |  | AP40.10-P-4054R |
| Tire rotation (where applicable) |  |  |
| Check thickness of brake pads and discs, front and rear wheels removed |  |  |
| pads |  | AP42.10-P-4252E |
| discs |  | AP42.10-P-4256EW |
| Visually inspect brake system components, lines, hoses, calipers |  |  |
| Inspect tires for damage and measure remaining tread life |  | AP40.10-P-4051EW |
| Check ex-factory driving aids (where applicable) |  | AP86.20-P-8649U |
| Check navigation system version for update |  |  |
| High Mileage Additional Work Performed (check if performed) |  |  |
| Rear Axle differential fluid replacement with CODE 467 (electronic locking differential) with CODE 471 (mechanical locking differential) with CODE P30 (AMG performance package) |  |  |
| with CODE 471 (mechanical locking differential) At 2,000 miles, then if more than 30,000 miles or 3 years since last replacement |  | AP35.00-P-3501CMG |
| with CODE 467 (electronic locking differential) At 2,000 miles, then if more than 30,000 miles or 3 years since last replacement |  | AP35.00-P-3501CMH |
| Replace air filter engine 177 | If more than 30,000 miles or 3years since last replacement | AP09.10-P-0980K |
| Replace air filter |  |  |
| engine 274, 276 |  |  |
| engine 274 | If more than 50,000 miles or 5 years since last replacement | AP09.10-P-0980CWG |
| engine 276 | If more than 50,000 miles or 5 years since last replacement | AP09.10-P-0980E |

| Replace spark plugs |  |  |
| --- | --- | --- |
| engine 177 | If more than 50,000 miles or 5 years since last replacement | AP15.10-P-1580C |
| engine 274 | If more than 50,000 miles or 5 years since last replacement | AP15.10-P-1580CWF |
| engine 276 | If more than 50,000 miles or 5 years since last replacement | AP15.10-P-1580D |
| Automatic transmission - oil and filter change engine 177 with transmission 725.09 (AMG models) | If more than 30,000 miles or 3years since last replacement | AP27.00-P-2702B |
| Automatic transmission - oil and filter change engine 274, 276 | If more than 60,000 miles or 6 years since last replacement | AP27.00-P-2702B |
| Replace fuel filter | If more than 120,000 miles or 12 years since last replacement | AP47.20-P-0780D |
| Replace main circuit coolant observe correct coolant composition | If more than 120,000 miles or 12 years since last replacement | AP20.00-P-2082G |
| Replace low-temperature circuit coolant |  |  |
| engine 177 | If more than 120,000 miles or 12 years since last replacement | AP20.00-P-2083C |
| engine 276 | If more than 120,000 miles or 12 years since last replacement | AP20.00-P-2083I |

| Tire inflation pressure (bar/psi) |  |  |  |  |
| --- | --- | --- | --- | --- |
| LF | RF | LR | RR | Spare |

| Tire thread depth (mm) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | LF | RF | LR | RR |
| outside |  |  |  |  |
| center |  |  |  |  |
| inside |  |  |  |  |