|  | Action | Result |
| --- | --- | --- |
| Prerequisites | Unplug diagnostic connector from vehicle |  |
|  | ABS malfunction light on |  |
|  | ESP warning lamp not lit |  |
|  | Flat surface, space to execute ¼ turn |  |
|  | Engine running |  |
|  | Steering straight |  |
|  | Parking brake off |  |
|  | Foot on brake |  |
|  | Transmission in Neutral |  |
|  | Release brake | Vehicle stationary (flat surface) |
| Sequence | Press & release ESP OFF switch | ESP warning light flashes then illuminates continuously |
|  | Press brake |  |
|  | Shift to Drive |  |
|  | Release brake |  |
|  | Within 10 seconds, accelerate to 10 km/h or 6 mph |  |
|  | Turn steering wheel minimum ¼ turn |  |
|  | Drive through semicircle | ESP warning light flashes |
|  | Apply brake to stop |  |
|  | Transmission to Neutral |  |
|  | Release brake | Vehicle stationary ABS warning light off ESP warning light off |
| Error Results | Test performed incorrectly | ABS warning light on |
|  | Fault in ESP control unit | ESP warning light on |
|  | (possible sensor short to ground or +12) |  |